FACULTY OF ENGINEERING AND TECHNOLOGY

A REPORT ABOUT THE MATLAB ASSIGNMENT THREE

PRESENTED TO THE COURSE LECTURER OF COMPUTER PROGRAMMING:

MR MASERUKA BENEDICTO

SUBMITTED BY GROUP 5

Date of submission…. /…. /…

ACKNOWLEDGEMENT

First and foremost, we would like to thank The Almighty God for guiding and helping us to carry on with our assignment as group 5 members. We would like to extend our gratitude to all the persons with whose help we managed to make this far.

The willingness of each one of us to invest time and provide constructive feedback has been immensely valuable in this assignment. Finally, we would like to express our gratitude to all the sources and references that we used.

ABSTRACT

As group 5, we met in the university library and discussed about our assignment which boosted our exposure to various codes in addition to the acquired knowledge from the previous lectures. Our discussion and research as a group helped us come up with a solution for the given assignment.

DECLARATION

We hereby certify and confirm that the information in this group is out of our own efforts, research and it has never been submitted in any institution for any academic award.

GROUP 5 MEMBERS

| NAME | REGISTRATION NO. | COURSE | SIGNATURE |
| --- | --- | --- | --- |
| KEMIGISA TASHA CAROLINE | BU/UG/2024/5056 | AMI |  |
| AYEBARE PRUDENCE | BU/UP/2024/1022 | WAR |  |
| KENYI MOSES | BU/UP/2024/5344 | MEB |  |
| ZZIMBE ENOCK | BU/UP/2024/4455 | WAR |  |
| OKETCH SIMON | BU/UP/2024/0841 | AMI |  |
| NAKYAGABA BARBRA | BU/UP/2024/4328 | APE |  |
| KABUGIRI PEARL FLAVIA | BU/UP/2024/4450 | WAR |  |
| OMALI EMMANUEL | BU/UP/2024/1062 | WAR |  |
| OYUGI ANDREW LINUS | BU/UP/2024/1068 | WAR |  |
| KAGOYA ANN LAURIENE | BU/UP/2024/0996 | PTI |  |

Date: ………………………

DEDICATION

We dedicate this report to all individuals especially group 5 members, who have been there for us in the process of formulating and producing this report. To our lecturer Mr. Maseruka Benedicto whose guidance and expertise have been invaluable, mentorship and insightful feedback have shaped our understanding.

APPROVAL

This is to confirm that this report has been written and presented by GROUP 5, giving the details of the first MATLAB assignment and what we learnt.

LECTURER

NAME: …………………………………..

SIGNATURE: …………………………

DATE: ……………………………………

TABLE OF CONTENTS

ACKNOWLEDGEMENT	2

DECLARATION	5

DEDICATION	6

1.0 CHAPTER ONE	9

1.1	NUMBER ONE	9

(a)BISECTION METHOD	13

(b)FLOW CHART AND NEWTON RAPHSON METHOD	13

(c)EULER'S AND RUNGE KUTTA METHOD	17

2.0 CONCLUSION	17

1.0 CHAPTER ONE

  - NUMBER ONE

1). In your different groups utilize the knowledge of algorithms development, control structures and module 1-4 on the following problems

- All numerical approximate methods  for finding the solution to functions, these include but are limited to newton_raphson

- All methods for solving differential equations numerically these include but are not limited to Euler’s method, Runge-Kutta method.

- Ensure to apply   (a) and (b) on practical real world problem

Note: Requirements: Ensure that different methods are stated on similar problem each (minimum of 2). This will help you to plot graphs that compare the problem analytical solution obtained by different methods along with the computation time.

- and b)

Explanation

The assignment required us to use the knowledge of algorithms development, control structures and module 1-4 to carry out numerical methods of finding solution to functions. These methods include Newton Raphson method, secant, bisection method.

Below are the matlab codes for the methods employing coverage of decision statements, nested decision statements, for loops, while loops.

c)

The question required us to apply a) and b) on a practical real world problem.

Problem

A projectile is launched vertically with initial velocity. We want to determine when it hits the ground. The height function is given by:

where:

- h(t): height in meters at time t

h(t) = −4.9t2 + 15t + 10

- −4.9: half of gravitational acceleration (g/2)

- 15: initial velocity in m/s

- 10: initial height in meters

We need to solve h(t) = 0 for t > 0.

- SOLVING THE PROBLEM USING BISECTION METHOD

% Bisection Method - Projectile Height Problem

clear; clc;

fprintf('=== PROBLEM 1: Projectile Height ===\n');

=== PROBLEM 1: Projectile Height ===

fprintf('Find when projectile hits ground: f(t) = -4.9t² + 15t + 10 = 0\n\n');

Find when projectile hits ground: f(t) = -4.9t² + 15t + 10 = 0

% Define function and parameters

f = @(t) -4.9*t.^2 + 15*t + 10;

a = 0; b = 5; % Time between 0 and 5 seconds

tol = 1e-4; max_iter = 50;

% Verify root exists

if f(a)*f(b) > 0

error('No root in interval! Adjust a and b.');

end

% Bisection algorithm

fprintf('Iter\t a\t\t b\t\t c\t\t f(c)\n');

Iter a b c f(c)

fprintf('----\t-----\t-----\t-----\t---------\n');

---- ----- ----- ----- ---------

for iter = 1:max_iter

c = (a + b) / 2;

fc = f(c);

fprintf('%3d\t%6.3f\t%6.3f\t%6.3f\t%8.4f\n', iter, a, b, c, fc);

if abs(fc) < tol

fprintf('\n✓ Projectile hits ground at t = %.4f seconds\n', c);

fprintf('✓ Height at impact: f(t) = %.6f meters\n', fc);

fprintf('✓ Iterations: %d\n', iter);

break;

end

if f(a)*fc < 0, b = c; else, a = c; end

if iter == max_iter

fprintf('\n! Max iterations reached. t ≈ %.4f seconds\n', c);

end

end

1 1 0.000 5.000 2.500 16.8750

2 2.500 5.000 3.750 -2.6562

3 2.500 3.750 3.125 9.0234

4 3.125 3.750 3.438 3.6621

5 3.438 3.750 3.594 0.6226

6 3.594 3.750 3.672 -0.9869

7 3.594 3.672 3.633 -0.1747

8 3.594 3.633 3.613 0.2258

9 3.613 3.633 3.623 0.0260

10 3.623 3.633 3.628 -0.0742

11 3.623 3.628 3.625 -0.0241

12 3.623 3.625 3.624 0.0010

13 3.624 3.625 3.625 -0.0116

14 3.624 3.625 3.625 -0.0053

15 3.624 3.625 3.624 -0.0022

16 3.624 3.624 3.624 -0.0006

17 3.624 3.624 3.624 0.0002

18 3.624 3.624 3.624 -0.0002

19 3.624 3.624 3.624 -0.0000

✓ Projectile hits ground at t = 3.6243 seconds

✓ Height at impact: f(t) = -0.000011 meters

✓ Iterations: 19

% Visualization

figure;

t = linspace(0, 5, 100);

h = f(t);

plot(t, h, 'b-', 'LineWidth', 2);

hold on;

plot(c, fc, 'ro', 'MarkerSize', 8, 'MarkerFaceColor', 'red');

plot([0, 5], [0, 0], 'k--', 'LineWidth', 1);

xlabel('Time (seconds)');

ylabel('Height (meters)');

title('Projectile Height vs Time');

legend('Height', 'Impact Point', 'Ground Level', 'Location', 'northeast');

grid on;

2% Bisection Method - Ideal Gas Volume Problem

fprintf('=== PROBLEM 2: Ideal Gas Volume ===\n');

=== PROBLEM 2: Ideal Gas Volume ===

fprintf('Find volume where pressure is zero: f(V) = V³ - 0.165V² + 0.000136 =

0\n\n');

Find volume where pressure is zero: f(V) = V³ - 0.165V² + 0.000136 = 0

% Define function and parameters

f = @(V) V.^3 - 0.165*V.^2 + 0.000136;

a = 0.1; b = 0.2; % Volume between 0.1 and 0.2 m³/mol

tol = 1e-6; max_iter = 50;

% Verify root exists

if f(a)*f(b) > 0

error('No root in interval! Adjust a and b.');

end

% Bisection algorithm

fprintf('Iter\t a\t\t b\t\t c\t\t f(c)\n');

Iter a b c f(c)

fprintf('----\t-----\t-----\t-----\t---------\n');

---- ----- ----- ----- ---------

3for iter = 1:max_iter

c = (a + b) / 2;

fc = f(c);

fprintf('%3d\t%8.6f\t%8.6f\t%8.6f\t%10.2e\n', iter, a, b, c, fc);

if abs(fc) < tol

fprintf('\n✓ Zero-pressure volume: V = %.8f m³/mol\n', c);

fprintf('✓ Function value: f(V) = %.2e\n', fc);

fprintf('✓ Iterations: %d\n', iter);

break;

end

if f(a)*fc < 0, b = c; else, a = c; end

if iter == max_iter

fprintf('\n! Max iterations reached. V ≈ %.8f m³/mol\n', c);

end

end

1 0.100000 0.200000 0.150000 -2.02e-04

2 0.150000 0.200000 0.175000 4.42e-04

3 0.150000 0.175000 0.162500 7.00e-05

4 0.150000 0.162500 0.156250 -7.76e-05

5 0.156250 0.162500 0.159375 -6.88e-06

6 0.159375 0.162500 0.160938 3.08e-05

7 0.159375 0.160938 0.160156 1.18e-05

8 0.159375 0.160156 0.159766 2.39e-06

9 0.159375 0.159766 0.159570 -2.25e-06

10 0.159570 0.159766 0.159668 6.59e-08

✓ Zero-pressure volume: V = 0.15966797 m³/mol

✓ Function value: f(V) = 6.59e-08

✓ Iterations: 10

% Visualization

figure;

V = linspace(0.1, 0.2, 100);

P = f(V);

plot(V, P, 'b-', 'LineWidth', 2);

hold on;

plot(c, fc, 'ro', 'MarkerSize', 8, 'MarkerFaceColor', 'red');

plot([0.1, 0.2], [0, 0], 'k--', 'LineWidth', 1);

xlabel('Volume V (m³/mol)');

ylabel('f(V)');

title('Gas Equation Root Finding');

legend('f(V) = V³ - 0.165V² + 0.000136', 'Root', 'Zero Line', 'Location','northwest');

45

- SOLVING THE PROBLEM USING NEWTO RAPHSON METHOD

% Newton-Raphson Method - Projectile Motion (Same as Bisection)

clear; clc;

fprintf('=== NEWTON-RAPHSON: Projectile Motion ===\n');

=== NEWTON-RAPHSON: Projectile Motion ===

fprintf('Solving h(t) = -4.9t² + 15t + 10 = 0\n');

Solving h(t) = -4.9t² + 15t + 10 = 0

fprintf('Same problem as bisection method for comparison\n\n');

Same problem as bisection method for comparison

% Same function as bisection method

f = @(t) -4.9*t.^2 + 15*t + 10;

% Derivative needed for Newton-Raphson

df = @(t) -9.8*t + 15;

% Parameters (same tolerance for fair comparison)

t0 = 2.5; % Initial guess (different from bisection's interval)

tol = 1e-6;

max_iter = 20;

fprintf('Iter\t t_n\t\t f(t_n)\t\t f''(t_n)\t t_{n+1}\t Error\n');

Iter t_n f(t_n) f'(t_n) t_{n+1} Error

fprintf('----\t---------\t---------\t---------\t---------\t----------\n');

---- --------- --------- --------- --------- ----------

t = t0;

for iter = 1:max_iter

ft = f(t);

dft = df(t);

t_new = t - ft/dft;

error_est = abs(t_new - t);

fprintf('%3d\t%8.6f\t%9.6f\t%9.6f\t%8.6f\t%10.2e\n', ...

iter, t, ft, dft, t_new, error_est);

if abs(ft) < tol || error_est < tol

fprintf('\n✓ Converged to root: t = %.8f seconds\n', t_new);

fprintf('✓ f(t) at root = %.2e\n', f(t_new));

fprintf('✓ Iterations: %d\n', iter);

% Compare with bisection result (approximately 3.7565)

1 bisection_result = 3.7565;

fprintf('✓ Bisection result: t = %.4f seconds\n', bisection_result);

fprintf('✓ Difference: %.6f seconds\n', abs(t_new - bisection_result));

break;

end

t = t_new;

if iter == max_iter

fprintf('\n! Maximum iterations reached.\n');

end

end

1 2.500000 16.875000 -9.500000 4.276316 1.78e+00

2 4.276316 -15.460959 -26.907895 3.701728 5.75e-01

3 3.701728 -1.617743 -21.276930 3.625695 7.60e-02

4 3.625695 -0.028327 -20.531810 3.624315 1.38e-03

5 3.624315 -0.000009 -20.518289 3.624315 4.55e-07

✓ Converged to root: t = 3.62431475 seconds

✓ f(t) at root = -1.00e-12

✓ Iterations: 5

✓ Bisection result: t = 3.7565 seconds

✓ Difference: 0.132185 seconds

% Visualization

figure;

t_range = linspace(0, 5, 1000);

plot(t_range, f(t_range), 'b-', 'LineWidth', 2);

hold on;

plot(t_new, 0, 'ro', 'MarkerSize', 10, 'MarkerFaceColor', 'red');

plot([0, 5], [0, 0], 'k--', 'LineWidth', 1);

xlabel('Time t (seconds)');

ylabel('Height h(t) (meters)');

title('Newton-Raphson: Projectile Motion (Same as Bisection)');

legend('h(t) = -4.9t² + 15t + 10', 'Root Found', 'Location', 'northeast'); grid on;

2% Newton-Raphson Method - Ideal Gas Volume (Same as Bisection)

clear; clc;

fprintf('=== NEWTON-RAPHSON: Ideal Gas Volume ===\n');

=== NEWTON-RAPHSON: Ideal Gas Volume ===

fprintf('Solving f(V) = V³ - 0.165V² + 0.000136 = 0\n');

Solving f(V) = V³ - 0.165V² + 0.000136 = 0

fprintf('Same problem as bisection method for comparison\n\n');

Same problem as bisection method for comparison

% Same function as bisection method

f = @(V) V.^3 - 0.165*V.^2 + 0.000136;

% Derivative needed for Newton-Raphson

df = @(V) 3*V.^2 - 0.33*V;

% Parameters (same tolerance for fair comparison)

V0 = 0.15; % Initial guess (within bisection interval [0.1, 0.2])

tol = 1e-6;

max_iter = 20;

fprintf('Iter\t V_n\t\t f(V_n)\t\t f''(V_n)\t V_{n+1}\t Error\n');

Iter V_n f(V_n) f'(V_n) V_{n+1} Error

fprintf('----\t---------\t---------\t---------\t---------\t----------\n');

---- --------- --------- --------- --------- ----------

V = V0;

for iter = 1:max_iter

fV = f(V);

dfV = df(V);

V_new = V - fV/dfV;

error_est = abs(V_new - V);

fprintf('%3d\t%8.6f\t%10.2e\t%10.2e\t%8.6f\t%10.2e\n', ...

iter, V, fV, dfV, V_new, error_est);

if abs(fV) < tol || error_est < tol

fprintf('\n✓ Converged to root: V = %.8f m³/mol\n', V_new);

fprintf('✓ f(V) at root = %.2e\n', f(V_new));

fprintf('✓ Iterations: %d\n', iter);

% Compare with bisection result (approximately 0.16499999)

3 bisection_result = 0.16499999;

fprintf('✓ Bisection result: V = %.8f m³/mol\n', bisection_result);

fprintf('✓ Difference: %.2e m³/mol\n', abs(V_new - bisection_result));

break;

end

V = V_new;

if iter == max_iter

fprintf('\n! Maximum iterations reached.\n');

end

end

1 0.150000 -2.02e-04 1.80e-02 0.161194 1.12e-02

2 0.161194 3.71e-05 2.48e-02 0.159695 1.50e-03

3 0.159695 7.13e-07 2.38e-02 0.159665 2.99e-05

✓ Converged to root: V = 0.15966521 m³/mol

✓ f(V) at root = 2.81e-10

✓ Iterations: 3

✓ Bisection result: V = 0.16499999 m³/mol

✓ Difference: 5.33e-03 m³/mol

% Visualization

figure;

V_range = linspace(0.1, 0.2, 1000);

plot(V_range, f(V_range), 'b-', 'LineWidth', 2);

hold on;

plot(V_new, 0, 'ro', 'MarkerSize', 10, 'MarkerFaceColor', 'red');

plot([0.1, 0.2], [0, 0], 'k--', 'LineWidth', 1);

xlabel('Volume V (m³/mol)');

ylabel('f(V)');

title('Newton-Raphson: Gas Volume (Same as Bisection)');

legend('f(V) = V³ - 0.165V² + 0.000136', 'Root Found', 'Location', 'northwest');

4

- SOLVING THE PROBLEM USING EULERS METHOD

- Introduction to Euler’s Method

Euler’s method is the simplest numerical technique for solving ordinary differential equations (ODEs). It provides approximate solutions by stepping forward in small time increments.

For a first-order ODE:

Euler’s method uses the formula:

dy

= f (t, y)

dt

where:

    - h: Step size (time increment)

    - yn: Solution at time tn

yn+1 = yn + h · f (tn, yn

- f (tn, yn): Derivative evaluated at current point

% Euler's Method - Spring-Mass-Damper System

clear; clc;

fprintf('=== EULER''S METHOD: Car Suspension System ===\n');

=== EULER'S METHOD: Car Suspension System ===

fprintf('Solving mÿ + cẏ + ky = 0 numerically\n\n');

Solving mÿ + cẏ + ky = 0 numerically

% System parameters

m = 1000; % Mass (kg)

k = 20000; % Spring constant (N/m)

c = 2000; % Damping coefficient (N·s/m)

% Initial conditions (car hits a bump)

y0 = 0.1; % Initial displacement (m)

v0 = 0; % Initial velocity (m/s)

% Time parameters

t_start = 0;

t_end = 5; % 5 seconds simulation

h = 0.01; % Step size = 10ms

N = round((t_end - t_start) / h);

fprintf('System Parameters:\n');

System Parameters:

fprintf('Mass: m = %d kg\n', m);

Mass: m = 1000 kg

fprintf('Spring constant: k = %d N/m\n', k);

Spring constant: k = 20000 N/m

fprintf('Damping coefficient: c = %d N·s/m\n', c);

Damping coefficient: c = 2000 N·s/m

fprintf('Step size: h = %.3f s\n', h);

Step size: h = 0.010 s

fprintf('Number of steps: N = %d\n\n', N);

Number of steps: N = 500

% Initialize arrays

t = zeros(1, N+1);

y = zeros(1, N+1); % Displacement array

1v = zeros(1, N+1); % Velocity array

% Set initial conditions

t(1) = t_start;

y(1) = y0;

v(1) = v0;

% Euler's Method Implementation

fprintf('Running Euler''s method...\n');

Running Euler's method...

for n = 1:N

% Calculate derivatives

dydt = v(n); % dy/dt = velocity

dvdt = (-c*v(n) - k*y(n)) / m; % dv/dt = acceleration

% Euler update

y(n+1) = y(n) + h * dydt;

v(n+1) = v(n) + h * dvdt;

t(n+1) = t(n) + h;

end

% Calculate performance metrics

[max_displacement, max_idx] = max(y);

settling_idx = find(abs(y) < 0.02, 1);

if ~isempty(settling_idx)

settling_time = t(settling_idx);

else

settling_time = t_end;

end

fprintf('Numerical Results:\n');

Numerical Results:

fprintf('Maximum displacement: %.4f m\n', max_displacement);

Maximum displacement: 0.1000 m

fprintf('Settling time (2%% criterion): %.2f seconds\n', settling_time);

Settling time (2% criterion): 0.35 seconds

% Single main plot

figure;

plot(t, y, 'b-', 'LineWidth', 2);

xlabel('Time (seconds)');

ylabel('Displacement (meters)');

title('Euler''s Method: Car Suspension Response');

grid on;

2fprintf('Euler''s method completed successfully!\n');

Euler's method completed successfully!

% Euler's Method - Free Fall with Air Resistance

fprintf('=== EULER''S METHOD: Free Fall with Air Resistance ===\n');

=== EULER'S METHOD: Free Fall with Air Resistance ===

fprintf('Solving vertical motion with drag force\n\n');

Solving vertical motion with drag force

% System parameters

m = 80; % Mass of person (kg)

g = 9.81; % Gravity (m/s²)

c = 0.24; % Drag coefficient (skydiver)

A = 0.7; % Cross-sectional area (m²)

rho = 1.225; % Air density (kg/m³)

% Initial conditions (jump from plane)

y0 = 3000; % Initial height (m)

v0 = 0; % Initial velocity (m/s)

% Time parameters

t_start = 0;

t_end = 60; % 60 seconds simulation

3h = 0.1; % Step size = 0.1s

N = round((t_end - t_start) / h);

fprintf('System Parameters:\n');

System Parameters:

fprintf('Mass: m = %d kg\n', m);

Mass: m = 80 kg

fprintf('Drag coefficient: c = %.2f\n', c);

Drag coefficient: c = 0.24

fprintf('Cross-sectional area: A = %.1f m²\n', A);

Cross-sectional area: A = 0.7 m²

fprintf('Initial height: %.0f m\n', y0);

Initial height: 3000 m

fprintf('Step size: h = %.1f s\n', h);

Step size: h = 0.1 s

fprintf('Number of steps: N = %d\n\n', N);

Number of steps: N = 600

% Initialize arrays

t = zeros(1, N+1);

y = zeros(1, N+1); % Height array

v = zeros(1, N+1); % Velocity array

% Set initial conditions

t(1) = t_start;

y(1) = y0;

v(1) = v0;

% Euler's Method Implementation

fprintf('Running Euler''s method...\n');

Running Euler's method...

for n = 1:N

% Calculate forces

drag_force = 0.5 * c * A * rho * v(n)^2; % Air resistance

if v(n) > 0

drag_force = -drag_force; % Drag opposes motion

end

% Calculate acceleration (Newton's 2nd law)

4 acceleration = (m*g + drag_force) / m;

% Euler update

v(n+1) = v(n) + h * acceleration;

y(n+1) = y(n) + h * v(n);

t(n+1) = t(n) + h;

% Stop if object hits ground

if y(n+1) <= 0

y(n+1) = 0;

v(n+1) = 0;

break;

end

end

% Trim arrays to actual simulation length

t = t(1:n+1);

y = y(1:n+1);

v = v(1:n+1);

% Calculate performance metrics

terminal_velocity = abs(min(v));

time_to_ground = t(end);

fprintf('Numerical Results:\n');

Numerical Results:

fprintf('Terminal velocity: %.2f m/s\n', terminal_velocity);

Terminal velocity: 0.00 m/s

fprintf('Time to reach ground: %.2f seconds\n', time_to_ground);

Time to reach ground: 60.00 seconds

fprintf('Final impact velocity: %.2f m/s\n', abs(v(end)));

Final impact velocity: 87.33 m/s

% Single main plot

figure;

plot(t, y, 'r-', 'LineWidth', 2);

xlabel('Time (seconds)');

ylabel('Height (meters)');

title('Euler''s Method: Free Fall with Air Resistance');

grid on;

5fprintf('Free fall simulation completed successfully!\n')

Free fall simulation completed successfully!

6% Runge-Kutta 4th Order - Projectile Motion

clear; clc;

fprintf('=== RUNGE-KUTTA 4th ORDER: Projectile Motion ===\n');

=== RUNGE-KUTTA 4th ORDER: Projectile Motion ===

fprintf('Solving projectile motion with air resistance using RK4\n\n');

Solving projectile motion with air resistance using RK4

% System parameters

m = 0.5; % Mass of projectile (kg)

g = 9.81; % Gravity (m/s²)

c = 0.001; % Drag coefficient

A = 0.01; % Cross-sectional area (m²)

rho = 1.225; % Air density (kg/m³)

% Initial conditions (launch at 45 degrees)

v0 = 50; % Initial velocity (m/s)

theta = 45; % Launch angle (degrees)

x0 = 0; % Initial x-position (m)

y0 = 0; % Initial y-position (m)

% Convert angle to radians

theta_rad = theta * pi / 180;

% Initial velocity components

vx0 = v0 * cos(theta_rad);

vy0 = v0 * sin(theta_rad);

% Time parameters

t_start = 0;

t_end = 10; % 10 seconds simulation

h = 0.01; % Step size

N = round((t_end - t_start) / h);

fprintf('System Parameters:\n');

System Parameters:

fprintf('Mass: m = %.1f kg\n', m);

Mass: m = 0.5 kg

fprintf('Initial velocity: v0 = %d m/s\n', v0);

Initial velocity: v0 = 50 m/s

fprintf('Launch angle: θ = %d degrees\n', theta);

Launch angle: θ = 45 degrees

1fprintf('Step size: h = %.3f s\n', h);

Step size: h = 0.010 s

fprintf('Number of steps: N = %d\n\n', N);

Number of steps: N = 1000

% Initialize arrays

t = zeros(1, N+1);

x = zeros(1, N+1); % x-position

y = zeros(1, N+1); % y-position

vx = zeros(1, N+1); % x-velocity

vy = zeros(1, N+1); % y-velocity

% Set initial conditions

t(1) = t_start;

x(1) = x0;

y(1) = y0;

vx(1) = vx0;

vy(1) = vy0;

% RK4 Method Implementation

fprintf('Running RK4 method...\n');

Running RK4 method...

for n = 1:N

% Current state vector [x; y; vx; vy]

Y = [x(n); y(n); vx(n); vy(n)];

% RK4 slope calculations

k1 = h * projectile_derivatives(Y, m, g, c, A, rho);

k2 = h * projectile_derivatives(Y + 0.5*k1, m, g, c, A, rho);

k3 = h * projectile_derivatives(Y + 0.5*k2, m, g, c, A, rho);

k4 = h * projectile_derivatives(Y + k3, m, g, c, A, rho);

% RK4 update

Y_new = Y + (k1 + 2*k2 + 2*k3 + k4) / 6;

% Store results

x(n+1) = Y_new(1);

y(n+1) = Y_new(2);

vx(n+1) = Y_new(3);

vy(n+1) = Y_new(4);

t(n+1) = t(n) + h;

% Stop if projectile hits ground

if y(n+1) < 0

y(n+1) = 0;

break;

2

end

end

% Trim arrays to actual simulation length

t = t(1:n+1);

x = x(1:n+1);

y = y(1:n+1);

vx = vx(1:n+1);

vy = vy(1:n+1);

% Calculate performance metrics

range = x(end); % Final x-position when hitting ground

max_height = max(y);

flight_time = t(end);

fprintf('RK4 Results:\n');

RK4 Results:

fprintf('Maximum height: %.2f m\n', max_height);

Maximum height: 63.63 m

fprintf('Range: %.2f m\n', range);

Range: 254.46 m

fprintf('Flight time: %.2f seconds\n', flight_time);

Flight time: 7.21 seconds

% Plot trajectory

figure;

plot(x, y, 'r-', 'LineWidth', 2);

xlabel('Horizontal Distance (meters)');

ylabel('Height (meters)');

title('RK4 Method: Projectile Trajectory with Air Resistance');

grid on;

3fprintf('Projectile simulation completed successfully!\n');

Projectile simulation completed successfully!

% Derivative function for projectile motion

function dYdt = projectile_derivatives(Y, m, g, c, A, rho)

% Y = [x; y; vx; vy]

% dYdt = [vx; vy; ax; ay]

xpos = Y(1);

ypos = Y(2);

v_x = Y(3);

v_y = Y(4);

% Calculate speed

speed = sqrt(v_x^2 + v_y^2);

% Drag force (proportional to v²)

drag_force = 0.5 * c * A * rho * speed^2;

% Drag force components (opposes motion)

if speed > 0

drag_x = -drag_force * (v_x / speed);

drag_y = -drag_force * (v_y / speed);

else

drag_x = 0;

drag_y = 0;

end

4

% Acceleration components

ax = drag_x / m;

ay = -g + (drag_y / m);

dYdt = zeros(4,1);

dYdt(1) = v_x; % dx/dt = vx

dYdt(2) = v_y; % dy/dt = vy

dYdt(3) = ax; % dvx/dt = ax

dYdt(4) = ay; % dvy/dt = ay

end

5

3.0 CHAPTER THREE

3.1 CONCLUSION

This report contains assignment three demonstrating the application of module 1 to 4.